# Deep groove ball bearing 6000, 6200, 6300, 6800, 6900series

### Deep groove ball bearing 6000,6200,6300,6800,6900series

We are a leading enterprise of bearings. The company lies in the south bank of Hangzhou bay, adjacent the highway of Shanghai、Hangzhou and Ningbo.It is 65 KM away from the airport of Ningbo; 60 KM from the International Container Quay of Ningbo. It is 320KM distances from the International airport of Shanghai; 300KM distances from the International Container dock of Shanghai. It is near to the Hangzhou bay -bridge. The Hangzhou bay-bridge is being built by them. Our company own brand are: KJYJ 、JHOK. The quality certificate of ISO9001 has been passed by us. We company has got the Export-Licence.
There are at least 5000 square meters building for manufacturing .There're total square are around 8500 square meters. The employees are more than 500.The main productions are small bearings. There are some advance automatic machines for production. Annual output are more than 40 millions. The productions are being sold all over the world by us.

We export bearings with good quality and low price.
Please contact us to know details.

Deep-groove ball bearings are the most popular of the antifriction bearing types. They are available in a broad range of sizes, from miniature ball bearings with outside diameter of less than 9mm to extra-large bearings that exceed 1, 500mm in diameter. Small and mid. Size bearings are also available in a variety of shield, seal and snap ring arrangements. Big ones normally are fitted with solid brass or steel cages.
SPECIFICATIONS:
1,Oringnal ,KJYJ  BEARINGS
2,ISO9001-2000 PASSED
3,Packing:Tube/Box
PRODUCTS RANGE:
6000SERIES: 6000-6016  OPEN,ZZ,2RS,NR,UG   
6200SERIES: 6200-6216  OPEN,ZZ,2RS,NR,UG
6300SERIES: 6300-6316  OPEN,ZZ,2RS,NR,UG
6800SERIES: 6800-6810  OPEN,ZZ,2RS,NR,UG
6900SERIES: 6900-6910  OPEN,ZZ,2RS,NR,UG
BEARINGS SPECIFICATIONS
1,MATERIAL: GCR15
2,GRADE:ABEC-1
3,NOISE GRADE:Z1V1
4,CLEANRANCE:C0,C2,C3,C4
5,HAND FEELING:WITH VERY SMOOTH FEELING 
BEARINGS APPLICATIONS
POWER TOOLS
MOTOR
MOTORCYCLE
PUMP
TEXTILE MACHINES
SPORT ENQUIPMENT
OFFICE ENQUIPMENT

Payment Terms: T/T, D/P AT SIGHT, L/C
HOPE WE CAN EASTBLISH GOOD BUSINESS RELATIONSHIP WITH YOU!!!

| ITEM | Shape Dimension(mm) |  |  |  |  |  |  |  | Steel Ball |  | Basic Load Rating(KN) |  | Max Speed (rpm) |  | Weight (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bore d |  | Quter Diameter D |  | Width B |  | Radius r min |  | Size mm | Quantity Number | Dynamic Cr | Static Cor | Grease | Oil |  |
|  | mm | inch | mm | inch | mm | inch | mm | inch |  |  |  |  |  |  |  |
| 6000 Series |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6000 | 10 | 0.3937 | 26 | 1.0236 | 8 | 0.3149 | 0.3 | 0.0236 | 4.763 | 7 | 4.58 | 1.98 | 20000 | 28000 | 0.018 |
| 6001 | 12 | 0.4724 | 28 | 1.1024 | 8 | 0.3149 | 0.3 | 0.0236 | 4.763 | 8 | 5.1 | 2.38 | 19000 | 26000 | 0.02 |
| 6002 | 15 | 0.5906 | 32 | 1.2598 | 9 | 0.3543 | 0.3 | 0.0236 | 4.763 | 9 | 5.58 | 2.85 | 18000 | 24000 | 0.026 |
| 6003 | 17 | 0.6693 | 35 | 1.378 | 10 | 0.3937 | 0.3 | 0.0236 | 4.763 | 10 | 6 | 3.25 | 17000 | 22000 | 0.036 |
| 6004 | 20 | 0.7874 | 42 | 1.6535 | 12 | 0.4724 | 0.6 | 0.0394 | 6.35 | 9 | 9.38 | 5.02 | 15000 | 19000 | 0.069 |
| 6005 | 25 | 0.9843 | 47 | 1.8504 | 12 | 0.4724 | 0.6 | 0.0394 | 6.35 | 10 | 10.1 | 5.85 | 13000 | 17000 | 0.075 |
| 6006 | 30 | 1.1811 | 55 | 2.1654 | 13 | 0.5118 | 1 | 0.0394 | 7.144 | 11 | 10.18 | 6.91 | 10000 | 14000 | 0.116 |
| 6007 | 35 | 1.3780 | 62 | 2.4409 | 14 | 0.5512 | 1 | 0.0433 | 7.938 | 11 | 12.47 | 8.66 | 9000 | 12000 | 0.155 |
| 6008 | 40 | 1.5748 | 68 | 2.6772 | 15 | 0.5906 | 1 | 0.0433 | 7.938 | 12 | 13.1 | 9.45 | 8500 | 11000 | 0185 |
| 6009 | 45 | 1.7717 | 75 | 2.9528 | 16 | 0.6299 | 1 | 0.0433 | 8.731 | 12 | 16.22 | 11.96 | 8000 | 10000 | 0.231 |
| 6010 | 50 | 1.9685 | 80 | 3.1496 | 16 | 0.6299 | 1 | 0.0433 | 8.731 | 13 | 16.94 | 12.95 | 7000 | 9000 | 0.25 |
| 6011 | 55 | 2.1654 | 90 | 3.5433 | 18 | 0.7086 | 1.1 | 0.0591 | 11 | 12 | 23.28 | 17.86 | 7000 | 7500 | 0.362 |
| 6012 | 60 | 2.3622 | 95 | 3.7401 | 18 | 0.7086 | 1.1 | 0.0591 | 11 | 13 | 24.35 | 19.35 | 6300 | 7500 | 0.385 |
| 6013 | 65 | 2.5591 | 100 | 3.937 | 18 | 0.7086 | 1.1 | 0.0591 | 11.1125 | 13 | 24.66 | 19.74 | 6000 | 7000 | 0.41 |
| 6014 | 70 | 2.7559 | 110 | 4.3307 | 20 | 0.7874 | 1.1 | 0.0591 | 12.303 | 13 | 29.68 | 24.2 | 5600 | 6700 | 0.575 |
| 6015 | 75 | 2.9528 | 115 | 4.5275 | 20 | 0.7874 | 1.1 | 0.0591 | 12.303 | 14 | 30.91 | 26.06 | 5300 | 6300 | 0.603 |
| 6016 | 80 | 3.1496 | 125 | 4.9212 | 22 | 0.8661 | 1.1 | 0.0787 | 13.494 | 14 | 36.57 | 31.36 | 5000 | 6000 | 0.821 |

| ITEM | Shape Dimension(mm) |  |  |  |  |  |  |  | Steel Ball |  | Basic Load Rating(KN) |  | Max Speed (rpm) |  | Weight (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bore d |  | Quter Diameter D |  | Width B |  | Radius r min |  | Size mm | Quantity Number | Dynamic Cr | Static Cor | Grease | Oil |  |
|  | mm | inch | mm | inch | mm | inch | mm | inch |  |  |  |  |  |  |  |
| 6200 Series |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6200 | 10 | 0.3937 | 30 | 1.11811 | 9 | 0.3543 | 0.6 | 0.0236 | 4.763 | 8 | 5.1 | 2.38 | 19000 | 26000 | 0.028 |
| 6201 | 12 | 0.4724 | 32 | 1.2598 | 10 | 0.3937 | 0.6 | 0.0236 | 5.953 | 7 | 6.82 | 3.05 | 18000 | 24000 | 0.034 |
| 6202 | 15 | 0.5906 | 35 | 1.3780 | 11 | 0.4331 | 0.6 | 0.0236 | 5.953 | 8 | 7.65 | 3.72 | 17000 | 22000 | 0.043 |
| 6203 | 17 | 0.6693 | 40 | 1.5748 | 12 | 0.4724 | 0.6 | 0.0236 | 6.747 | 8 | 9.58 | 4.47 | 16000 | 20000 | 0.062 |
| 6204 | 20 | 0.7874 | 47 | 1.8504 | 14 | 0.5512 | 1.0 | 0.0394 | 7.938 | 8 | 9.88 | 6.2 | 14000 | 18000 | 0.102 |
| 6205 | 25 | 0.9843 | 52 | 2.0472 | 15 | 0.5906 | 1.0 | 0.0394 | 7.938 | 9 | 10.78 | 6.98 | 12000 | 16000 | 0.12 |
| 6206 | 30 | 1.1811 | 62 | 2.4409 | 16 | 0.6299 | 1.0 | 0.0394 | 9.525 | 9 | 14.97 | 10.04 | 9500 | 13000 | 0.19 |
| 6207 | 35 | 1.3780 | 72 | 2.8346 | 17 | 0.6693 | 1.1 | 0.0433 | 11.1125 | 9 | 19.75 | 13.67 | 8500 | 11000 | 0.27 |
| 6208 | 40 | 1.5748 | 80 | 3.1496 | 18 | 0.7087 | 1.1 | 0.0433 | 12 | 9 | 22.71 | 15.94 | 8000 | 10000 | 0.37 |
| 6209 | 45 | 1.7717 | 85 | 3.3465 | 19 | 0.7480 | 1.1 | 0.0433 | 12 | 10 | 24.36 | 17.71 | 7000 | 9000 | 0.38 |
| 6210 | 50 | 1.9685 | 90 | 3.5433 | 20 | 0.7874 | 1.1 | 0.0433 | 12.7 | 10 | 26.98 | 19.84 | 6700 | 8500 | 0.45 |
| 6211 | 55 | 2.1654 | 100 | 3.9370 | 21 | 0.8268 | 1.5 | 0.0591 | 14.288 | 10 | 33.37 | 25.11 | 6000 | 7500 | 0.603 |
| 6212 | 60 | 2.3622 | 110 | 4.3307 | 22 | 0.8661 | 1.5 | 0.0591 | 15.081 | 10 | 36.74 | 27.97 | 5600 | 7000 | 0.789 |
| 6213 | 65 | 2.5591 | 120 | 4.7244 | 23 | 0.9055 | 1.5 | 0.0591 | 16.669 | 10 | 44.01 | 34.18 | 5000 | 6300 | 0.99 |
| 6214 | 70 | 2.7559 | 125 | 4.9213 | 24 | 0.9449 | 1.5 | 0.0591 | 16.669 | 11 | 46.79 | 37.59 | 4800 | 6000 | 1.084 |
| 6215 | 75 | 2.9528 | 130 | 5.1181 | 25 | 0.9843 | 1.5 | 0.0591 | 17.462 | 11 | 66 | 49.5 | 4800 | 5600 | 1.18 |
| 6216 | 80 | 3.1496 | 140 | 5.5118 | 26 | 1.0236 | 2.0 | 0.0787 | 18.256 | 11 | 72.5 | 53 | 4500 | 5300 | 1.4 |

| ITEM | Shape Dimension(mm) |  |  |  |  |  |  |  | Steel Ball |  | Basic Load Rating(KN) |  | Max Speed (rpm) |  | Weight (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bore d |  | Quter Diameter D |  | Width B |  | Radius r min |  | Size mm | Quantity Number | Dynamic Cr | Static Cor | Grease | Oil |  |
|  | mm | inch | mm | inch | mm | inch | mm | inch |  |  |  |  |  |  |  |
| 6300 Series |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6300 | 10 | 0.3937 | 35 | 1.378 | 11 | 0.4331 | 0.6 | 0.0236 | 6.35 | 7 | 5.88 | 3.47 | 20000 | 26000 | 0.054 |
| 6301 | 12 | 0.4724 | 37 | 1.4567 | 12 | 0.4724 | 1 | 0.0394 | 7.938 | 6 | 7.48 | 4.65 | 17000 | 22000 | 0.058 |
| 6302 | 15 | 0.5906 | 42 | 1.6536 | 13 | 0.5118 | 1 | 0.0394 | 7.938 | 7 | 8.8 | 5.43 | 16000 | 20000 | 0.082 |
| 6303 | 17 | 0.6693 | 47 | 1.8504 | 14 | 0.5512 | 1 | 0.0394 | 8.731 | 7 | 10.45 | 6.5 | 15000 | 19000 | 0.11 |
| 6304 | 20 | 07874 | 52 | 2.0472 | 15 | 0.5906 | 1.1 | 0.0433 | 9.525 | 7 | 12.26 | 7.81 | 13000 | 17000 | 0.14 |
| 6305 | 25 | 0.9843 | 62 | 2.4409 | 17 | 0.6693 | 1.1 | 0.0433 | 11.5 | 7 | 17.22 | 11.39 | 10000 | 14000 | 0.22 |
| 6306 | 30 | 1.1811 | 72 | 2.8346 | 19 | 0.748 | 1.1 | 0.0433 | 12 | 8 | 20.77 | 14.17 | 9000 | 12000 | 0.33 |
| 6307 | 35 | 1.378 | 80 | 3.1496 | 21 | 0.8268 | 1.5 | 0.0591 | 13.494 | 8 | 25.66 | 17.92 | 8000 | 10000 | 0.41 |
| 6308 | 40 | 1.5784 | 90 | 3.5433 | 23 | 0.9056 | 1.5 | 0.0591 | 15.081 | 8 | 31.35 | 22.38 | 7000 | 9000 | 0.6 |
| 6309 | 45 | 1.7717 | 100 | 3.937 | 25 | 0.9843 | 1.5 | 0.0591 | 17.462 | 8 | 40.66 | 30 | 6300 | 7500 | 0.77 |
| 6310 | 50 | 1.9685 | 110 | 4.3307 | 27 | 1.063 | 2 | 0.0788 | 19.05 | 8 | 47.58 | 35.71 | 6000 | 7000 | 1.03 |
| 6311 | 55 | 2.1653 | 120 | 4.7244 | 29 | 1.1417 | 2 | 0.0788 | 20.638 | 8 | 55.05 | 41.91 | 5600 | 6700 | 1.367 |
| 6312 | 60 | 2.3622 | 130 | 5.1181 | 31 | 1.2205 | 2.1 | 0.0827 | 22.225 | 8 | 62.88 | 48.6 | 5000 | 6000 | 1.71 |
| 6313 | 65 | 2.59 | 140 | 5.5118 | 33 | 1.2992 | 2.1 | 0.0827 | 24 | 8 | 72.21 | 56.68 | 4500 | 5300 | 2.1 |
| 6314 | 70 | 2.7559 | 150 | 5.9055 | 35 | 1.378 | 2.1 | 0.0827 | 25.4 | 8 | 80.1 | 63.48 | 4300 | 5000 | 2.55 |

| ITEM | Shape Dimension(mm) |  |  |  |  |  |  |  |  | Steel Ball |  |  | Basic Load Rating(KN) |  | Max Speed (rpm) |  | Weight (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bore d |  | Quter Diameter D |  | Width B |  |  | Radius r min |  | Size mm |  | Quantity Number | Dynamic Cr | Static Cor | Grease | Oil |  |
|  | mm | inch | mm | inch | mm | inch |  | mm | inch |  |  |  |  |  |  |  |  |
| 6800 Series |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6800 | 10 | 0.3937 | 19 | 0.7480 | 5 | 0.1969 | 0.3 |  | 0.0118 | 2.381 | 9 |  | 1.83 | 2.381 | 26000 | 34000 | 0.005 |
| 6801 | 12 | 0.4724 | 21 | 0.8268 | 5 | 0.1969 | 0.3 |  | 0.0118 | 2.381 | 12 |  | 1.92 | 2.381 | 22000 | 30000 | 0.006 |
| 6802 | 15 | 0.5906 | 24 | 0.9449 | 5 | 0.1969 | 0.3 |  | 0.0118 | 2.381 | 14 |  | 2.08 | 2.381 | 20000 | 28000 | 0.007 |
| 6803 | 17 | 0.6693 | 26 | 1.0236 | 5 | 0.1969 | 0.3 |  | 0.0118 | 2.381 | 15 |  | 2.81 | 2.381 | 19000 | 26000 | 0.008 |
| 6804 | 20 | 0.7874 | 32 | 1.2598 | 7 | 0.2756 | 0.3 |  | 0.0118 | 3.5 | 13 |  | 4 | 3.5 | 17000 | 22000 | 0.019 |
| 6805 | 25 | 0.9843 | 37 | 1.4567 | 7 | 0.2756 | 0.3 |  | 0.0118 | 3.5 | 15 |  | 4.3 | 3.5 | 15000 | 19000 | 0.022 |
| 6806 | 30 | 1.1811 | 42 | 1.6535 | 7 | 0.2756 | 0.3 |  | 0.0118 | 3.175 | 19 |  | 4.7 | 3.175 | 13000 | 17000 | 0.026 |
| 6807 | 35 | 1.3780 | 47 | 1.8504 | 7 | 0.2756 | 0.3 |  | 0.0118 | 3.175 | 22 |  | 4.9 | 3.175 | 11000 | 15000 | 0.029 |
| 6808 | 40 | 1.5784 | 52 | 2.0472 | 7 | 0.2756 | 0.3 |  | 0.0118 | 3.175 | 22 |  | 5.1 | 3.175 | 9000 | 13000 | 0.033 |
| 6809 | 45 | 1.7717 | 58 | 2.2835 | 7 | 0.2756 | 0.3 |  | 0.0118 | 3.175 | 24 |  | 6.4 | 3.175 | 7000 | 11000 | 0.04 |
| 6810 | 50 | 1.9685 | 65 | 2.5591 | 7 | 0.2756 | 0.3 |  | 0.0118 | 3.175 | 24 |  | 6.6 | 3.175 | 5000 | 9000 | 0.052 |

| ITEM | Shape Dimension(mm) |  |  |  |  |  |  |  | Steel Ball |  | Basic Load Rating(KN) |  | Max Speed (rpm) |  | Weight (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bore d |  | Quter Diameter D |  | Width B |  | Radius r min |  | Size mm | Quantity Number | Dynamic Cr | Static Cor | Grease | Oil |  |
|  | mm | inch | mm | inch | mm | inch | mm | inch |  |  |  |  |  |  |  |
| 6900 Series |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6900 | 10 | 0.3937 | 22 | 0.8661 | 6 | 0.2362 | 0.3 | 0.0118 | 2.381 | 12 | 2.7 | 1.27 | 25000 | 32000 | 0.009 |
| 6901 | 12 | 0.4724 | 24 | 0.9449 | 6 | 0.2362 | 0.3 | 0.0118 | 3.5 | 9 | 2.89 | 1.46 | 20000 | 28000 | 0.011 |
| 6902 | 15 | 0.5906 | 28 | 1.1024 | 6 | 0.2362 | 0.3 | 0.0118 | 3.969 | 10 | 4.1 | 2.06 | 19000 | 26000 | 0.016 |
| 6903 | 17 | 0.6693 | 30 | 1.1811 | 7 | 0.2756 | 0.3 | 0.0118 | 3.969 | 11 | 4.65 | 2.58 | 18000 | 24000 | 0.018 |
| 6904 | 20 | 0.7874 | 37 | 1.4567 | 7 | 0.2756 | 0.3 | 0.0118 | 4.763 | 11 | 6.4 | 3.7 | 17000 | 22000 | 0.036 |
| 6905 | 25 | 0.9843 | 42 | 1.6535 | 9 | 0.3543 | 0.3 | 0.0118 | 4.763 | 13 | 7.05 | 4.56 | 14000 | 18000 | 0.042 |
| 6906 | 30 | 1.1811 | 47 | 1.8504 | 9 | 0.3543 | 0.3 | 0.0118 | 4.763 | 14 | 7.25 | 5 | 12000 | 16000 | 0.048 |
| 6907 | 35 | 1.3780 | 55 | 2.1654 | 10 | 0.3937 | 0.3 | 0.0118 | 5.953 | 13 | 11.2 | 7.45 | 10000 | 14000 | 0.074 |
| 6908 | 40 | 1.5784 | 62 | 2.4409 | 12 | 0.4724 | 0.3 | 0.0118 | 6.747 | 14 | 14.5 | 10.2 | 8000 | 12000 | 0.11 |
| 6909 | 45 | 1.7717 | 68 | 2.6772 | 12 | 0.4724 | 0.3 | 0.0118 | 6.747 | 14 | 15.1 | 11.2 | 6000 | 10000 | 0.128 |
| 6910 | 50 | 1.9685 | 72 | 2.8346 | 12 | 0.4724 | 0.3 | 0.0118 | 6.747 | 15 | 15.6 | 12.2 | 4000 | 8000 | 0.132 |

Related Keywords: Deep Groove Ball Bearing  Double Row Deep Groove Ball Bearing  Miniature Deep Groove Ball Bearings

Email: andyfung.china@gmail.com ；asiabearings@gmail.com

Homepage: www.asiabearings.com